# Control of Substances Hazardous to Health (COSHH) Risk Assessment Form

| Name of substance: | Sprint Cream Cleaner |
| --- | --- |
| Other documents to be referenced using this substance: | Refer to Safety Data Sheet. |
| Assessment date: | 05/05/2022 |
| Review date: | 05/05/2025 |
| Describe the activity or work process: (Include how long and how often this is carried out and the quantity of substance used) | General purpose cleaner – hand applied onto surface with cloth |
| Identify the persons at risk: (Employees, Contractors, Public, Other – please specify) | Employees Contractors Members of Public |
| Name of the manufacturer/supplier: | Diversey ltd. |
| Concentration: | Calcium carbonate 30-50% Sulphonic acids, C14-17-sec-alkane, sodium salta 3-10% Alkyl alcohol ethoxylate 1-3% Gultaral 0.01-0.1% 1,2-benzisithiazol-3(2H)-one <0.01 |

(A copy of a current safety data sheet for this substance must be attached to this assessment.)

Contents

Classification (Category of Danger)	1

Personal Protective Equipment (PPE)	2

First Aid and Emergency Measures	3

Storage, Handling, and Other Information	3

Signed	4

# Classification (Category of Danger)

| Danger | Symbol | Yes/No/Not Applicable (N/A) |
| --- | --- | --- |
| Flammable |  |  |
| Oxidising |  |  |
| Explosive |  |  |
| Irritant/Harmful |  | Yes |
| Environment |  |  |
| Carcinogenic |  |  |
| Corrosive |  |  |
| Toxic |  |  |
| Pressurised |  |  |
| Other – please specify |  |  |

Hazard/Precautionary Statements (must be included)

Signal Word: WARNING H319: Causes serious eye irritation.

| Hazard Type | Yes/No/Not Applicable (N/A) |
| --- | --- |
| Gas |  |
| Vapour |  |
| Dust |  |
| Mist |  |
| Fume |  |
| Dust |  |
| Liquid | Yes |
| Solid |  |
| Other – please specify |  |

| Route of Exposure | Yes/No/Not Applicable (N/A) |
| --- | --- |
| Inhalation | Yes |
| Skin | Yes |
| Eyes | Yes |
| Ingestion | Yes |
| Injection |  |
| Other – please specify |  |

Workplace Exposure Limits (WELs) - Please indicate if not applicable (N/A)

| Short-term exposure (15 minutes) | Calcium carbonate: Inhalable dust: 30mg/m3 Respirable dust: 12mg/m3 Glutaral: 0.05 ppm/0.2 mg/m3 |
| --- | --- |
| Long-term exposure level (8 hour Time Weighted Average (TWA)) | Calcium carbonate: Inhalable dust: 10mg/m3 Respirable dust: 4mg/m3 Glutaral: 0.05 ppm/0.2 mg/m3 |

# Personal Protective Equipment (PPE)

| Item | Symbol | If yes – specify type/standard |
| --- | --- | --- |
| Air Fed Helmet |  |  |
| Face Visor |  |  |
| Goggles |  |  |
| Safety Glasses |  | Yes – EN166 if splashes are likely |
| Safety Boots/Shoes |  |  |
| Hand Protection |  | Yes – EN374 Nitrile |
| Overall |  |  |
| Apron |  |  |
| Lab Coat |  |  |
| Half Respirator |  |  |
| Full Respirator |  |  |
| Face Mask |  |  |
| Ventilation |  |  |
| Local Exhaust Ventilation |  |  |
| Used Outside |  |  |
| Other – please specify |  |  |

# First Aid and Emergency Measures

| Contact Type | Description of First Aid Measures |
| --- | --- |
| Inhaled | Get medical attention or advice if you feel unwell. |
| Skin Contact | Wash skin with plenty of lukewarm, gently flowing water. If skin irritation occurs: Get medical advice or attention. |
| Eye Contact | Immediately rinse eyes cautiously with lukewarm water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. If irritation occurs and persists, get medical attention. |
| Swallowed | Immediately drink 1 glass of water. Get medical attention or advice if you feel unwell. |

| Emergency Measures | Yes/No/Not Applicable (N/A) |
| --- | --- |
| Is health surveillance required? | No |
| Is training required for this process? | No |
| Fire precautions: What actions will be taken in the event of a fire involving these substance(s)? | Extinguishing media: Carbon dioxide. Dry powder. Water spray jet. Fight larger fires with water spray jet or alcohol-resistant foam. As in any fire, wear self contained breathing apparatus and suitable protective clothing including gloves and eye/face protection. |
| How should an accidental release/spillage be dealt with? | Do not allow to enter drainage system, surface or ground water. Dilute with plenty of water. Absorb with liquid-binding material (sand, diatomite, universal binders, sawdust). |

# Storage, Handling, and Other Information

| Handling | Handle in accordance with good industrial hygiene and safety practice. Keep away from food, drink and animal feeding stuffs. Do not mix with other products unless adviced by Sealed Air. Wash hands before breaks and at the end of workday. Wash face, hands and any exposed skin thoroughly after handling. Take off immediately all contaminated clothing. Use personal protective equipment as required. Use only with adequate ventilation. |
| --- | --- |
| Storage | Store in accordance with local and national regulations. Keep only in original container. Store in a closed container. |

| Disposal Method – How should these substances be disposed of (or not disposed of?) | Yes/No/Not Applicable (N/A) |
| --- | --- |
| Hazardous waste |  |
| Skip |  |
| Return to depot |  |
| Return to supplier |  |
| Other (please state) | The concentrated contents or contaminated packaging should be disposed of by a certified handler or according to the site permit. Release of waste to sewers is discouraged. The cleaned packaging material is suitable for energy recovery or recycling in line with local legislation. |

# Signed

| Assessment completed by: | The CoSHH Group |
| --- | --- |
| Signature: | The CoSHH Group |
| Date: | 05/05/2022 |

Return to top of document